aaa-policy-control (PCEF Profile)

date_range 01-Oct-25

Syntax

aaa-policy-control {

aaa-profile aaa-profile-name;

pcc-rulebases [rulebase-name];

user-password password;

}

Hierarchy Level

[edit unified-edge pcef profiles profile-name]

Description

Configure RADIUS-server-controlled policy management for a policy and charging enforcement function (PCEF) profile.

The remaining statements are explained separately.

Required Privilege Level

unified-edge—To view this statement in the configuration.

unified-edge-control—To add this statement to the configuration

ssh (NETCONF)

date_range 20-Nov-23

Syntax

ssh {

client-alive-count-max number;

client-alive-interval seconds;

connection-limit limit;

port port-number;

rate-limit limit;

}

Hierarchy Level

[edit system services netconf]

Description

Enable access to the NETCONF SSH subsystem using the default port number 830, as specified by RFC 4742.

Options

| client-alive-count-max number | (Optional) Threshold of client-alive responses that can be missed before the sshd process disconnects the client, thereby terminating the NETCONF session. Use this statement in conjunction with the client-alive-interval statement to disconnect unresponsive NETCONF clients. - Default: 3 - Range: 0 through 255 |
| --- | --- |
| client-alive-inteval seconds | (Optional) Timeout interval in seconds, after which, if no data has been received from the client, the sshd process sends a message through the encrypted channel to request a response from the client. Use this statement in conjunction with the client-alive-count-max statement to disconnect unresponsive NETCONF clients. This option applies to SSH protocol version 2 only. - Default: 0 seconds - Range: 0 through 65535 seconds |

authentication-sha256

date_range 01-Oct-25

Syntax

authentication-sha256 {

authentication-key authentication-key;

authentication-password authentication-password;

}

Hierarchy Level

[edit snmp v3 usm local-engine user username],

[edit snmp v3 usm remote-engine engine-id user username]

Description

Configure the secure hash algorithm (SHA)256 as the authentication type for the SNMPv3 user. You can use this configuration statement for local and remote user.

Required Privilege Level

snmp—To view this statement in the configuration.

snmp-control—To add this statement to the configuration.

logging (Services)

Syntax

logging {

traceoptions {

file filename <files number> <match regular-expression> <size size> <world-readable | no-world-readable>;

flag flag;

no-remote-trace;

}

}

Hierarchy Level

[edit services]

Description

CAUTION:Enabling tracing can adversely impact scale and performance and may increase security risk. We strongly recommend using the trace, tracing, or traceoptions commands only under the guidance of a JTAC support engineer. After collecting the debug information, immediately disable tracing to minimize risk and restore normal system performance.

Define global services properties.

The "traceoptions" section controls the collection of diagnostic trace information. It can specify the trace output file, number and size of trace files, filtering using regular expressions, trace flags, file access permissions, and whether remote tracing is allowed.

Important options:

- file: Specifies the file where trace information is stored.

- files: Specifies the number of trace files to maintain.

- size: Specifies the maximum size of a trace file.

- match: Filters trace information using a regular expression.

- flag: Specifies the type of information or events to trace.

- world-readable: Allows all users to read the trace file.

- no-world-readable: Restricts access to the trace file.

- no-remote-trace: Prevents tracing from being performed remotely.

Security significance:

Tracing is primarily a diagnostic and troubleshooting mechanism rather than a general security event logging mechanism. Juniper warns that enabling tracing can negatively affect system scale and performance and may increase security risk. Tracing should therefore be enabled only when necessary and disabled after the required diagnostic information has been collected.

When analyzing a Juniper configuration, the presence of [edit services] logging or traceoptions should be interpreted as service-level diagnostic tracing. It should not by itself be considered evidence that security event logging, audit logging, or continuous system monitoring is properly configured.

Options

The remaining statement is explained separately. See CLI Explorer.

Required Privilege Level

interface—To view this statement in the configuration.

interface-control—To add this statement to the configuration.

Juniper Junos Services Logging

authentication (AAA Profile)

Syntax

network-element network-element-name;

}

Hierarchy Level

[edit unified-edge aaa profiles aaa-profile-name radius]

Description

Specify the network element providing policy management for TDF subscribers. The network element must already be defined at the [edit access radius] hierarchy level.

The remaining statement is explained separately. See CLI Explorer.

Required Privilege Level

unified-edge—To view this statement in the configuration.

unified-edge-control—To add this statement to the configuration.

authorization-order

Syntax

authorization-order [ authorization-methods ];

Hierarchy Level

[edit access profile profile-name]

Description

Set the order in which AAA tries different methods to verify that a client is authorized access to the router or switch. For each login attempt, AAA tries the authorization methods in order, from first to last.

A given subscriber does not undergo both authentication and authorization as separate steps. When both authentication-order and authorization-order are specified, DHCP subscribers honor the configured authorization order, all other subscribers use the configured authentication-order.

Options

| authorization-methods | Ordered list of methods to use for authorization attempts. The list includes one or more of the following methods in any combination: - jsrc—Use the JSRC application in an SRC environment to request authorization from the SAE when verifying that a subscriber can access the router or switch. - When you configure both this option and authentication-order, AAA ignores the authentication order setting for DHCP subscribers. For non-DHCP subscribers, AAA ignores the authorization order and applies only the authentication order. - When you configure only this option, AAA applies the authorization order to both DHCP and non-DHCP subscribers. - nasreq—Use the NASREQ application to communicate with a NASREQ server for authorization of any subscriber type as an alternative to RADIUS authorization. - none—No authorization is performed. Can be used, for example, when the Diameter function Gx-Plus is employed for notification during subscriber provisioning. |
| --- | --- |

http (Access)

Syntax

http {

banner {

fail string;

login string;

success string;

}

}

Hierarchy Level

[edit access firewall-authentication pass-through]

Description

Configure banners for the HTTP login prompt, successful authentication, and failed authentication.

Options

The remaining statements are explained separately. See CLI Explorer.

banner (Access FTP HTTP Telnet Authentication)

Syntax

content_copy zoom_out_map

banner {

fail string;

login string;

success string;

}

Hierarchy Level

content_copy zoom_out_map

[edit access firewall-authentication pass-through (ftp | http | telnet)]

Description

Configure the banners that appear to users during the FTP, HTTP, HTTPS, and Telnet firewall authentication process. The banners appear during login, after successful authentication, and after failed authentication.

Options

The remaining statements are explained separately. See CLI Explorer.

Required Privilege Level

access—To view this statement in the configuration.

access-control—To add this statement to the configuration.

Release Information

Statement introduced in Junos OS Release 8.5.

HTTPS for Web authentication is supported on SRX5400, SRX5600, and SRX5800 devices and SRX Series Firewalls from Junos OS Release 12.1X44-D10 and on SRX300, SRX320, SRX340, SRX345, SRX380, and SRX1500 Services Gateways from Junos OS Release 15.1X49-D40.

ntp

Syntax

ntp {

authentication-key key-number type (md5 | sha1 | sha256) value password;

boot-server  (address | hostname);

broadcast <address> <key key-number> <routing-instance-name routing-instance-name> <ttl value>  <version value>;

broadcast-client;

interval-range value;

multicast-client <address>;

nts <local-certificate local-certificate><trusted-ca (trusted-ca-group trusted-ca-group | trusted-ca-profile trusted-ca-profile)>;

peer address <key key-number> <prefer> <version value>;

restrict address {

mask network-mask;

noquery;

}

server name {

key key;

nts <remote-identity distinguished-name(container container | wildcard wildcard) hostname hostname>;

prefer;

routing-instance routing-instance;

version version;

}

source-address source-address <routing-instance routing-instance-name>;

threshold value action (accept | reject);

trusted-key [ key-numbers ];

}

Hierarchy Level

[edit system]

Description

Configure NTP on the device. In both standalone and chassis cluster modes, the primary Routing Engine runs the NTP process to get the time from the external NTP server. Although the secondary Routing Engine runs the NTP process in an attempt to get the time from the external NTP server, this attempt fails because of network issues. For this reason, the secondary Routing Engine uses NTP to get the time from the primary Routing Engine.

When configuring the NTP service in the management VRF (mgmt_junos), you must configure at least one IP address on a physical or logical interface within the default routing instance and ensure that this interface is up in order for the NTP service to work with the mgmt_junos VRF.

| authentication-key key_number | Configure key (key ID, key type, and key value) to authenticate NTP packets with the devices (servers and clients). The authentication key has two fields: - type—When authentication is specified, the key identifier (key ID) followed by the message digest is appended to the NTP packet header. The supported message digest formats are md5, sha1, sha256. NOTE: - For sha256 message digest format note the following aspects: - Keys with less than or equal to 20 characters are treated as ASCII strings - Keys with more than 20 characters are treated as hexadecimal strings - Keys with more than 20 characters and containing invalid hexadecimal characters are skipped with an error - EX4600 does not support SHA256 authentication for NTP and supports MD5 only. - value—If the key value is available in ASCII format and without special characters, it can be entered directly. If the key value contains special characters or is available in hex format, consider the following: For specifying the keys in hex format, prepend a "\x" for each two characters. For hex key example, af60112f...39af4ced, set system ntp authentication-key <ID> value "\xaf\x60\x11\x2f\....\x39\xaf\x4c\xed". If the key contains one of the characters from (null) 0x00, (space) 0x20, " 0x22, & 0x26, ( 0x28 ) 0x29 prepend a "\\x" . For example, \\x22. - Range: 1 to 65534 |
| --- | --- |

snmp (SNMP Scripts)

Syntax

snmp {

file filename {

cert-file path;

checksum hash-algorithm hash-value;

oid id {

priority;

}

python-script-user username;

refresh;

refresh-from;

routing-instance routing-instance-name;

source;

start {

start-options {

mem-factor percentage;

}

}

}

max-datasize size;

refresh;

refresh-from;

start {

start-options {

mem-factor percentage;

}

}

traceoptions {

file <filename> <files number> <size size> <world-readable | no-world-readable>;

flag flag;

no-remote-trace;

}

}

Hierarchy Level

[edit system scripts]

Description

For Junos OS SNMP scripts, configure the SNMP scripting mechanism.

The remaining statements are explained separately. See CLI Explorer.

traceoptions (SNMP)

Syntax

content_copy zoom_out_map

traceoptions {

file filename <files number> <match regular-expression> <size size> <world-readable | no-world-readable>;

flag flag;

no-remote-trace;

}

Hierarchy Level

content_copy zoom_out_map

[edit snmp]

Description

CAUTION:

Enabling tracing can adversely impact scale and performance and may increase security risk. We strongly recommend using the trace, tracing, or traceoptions commands only under the guidance of a JTAC support engineer. After collecting the debug information, immediately disable tracing to minimize risk and restore normal system performance.

Track the activities of SNMP agents on the switch and record the information in log files.

NOTE: The traceoptions statement is not supported on the QFabric system.

The output of the tracing operations is placed into log files in the /var/log directory. Each log file is named after the SNMP agent that generates it. Currently, the following logs are created in the /var/log directory when the traceoptions statement is used:

- chassisd

- craftd

- ilmid

- mib2d

- rmopd

- serviced

- snmpd

Options

file filename—By default, the name of the log file that records trace output is the name of the process being traced (for example, mib2d or snmpd). Use this option to specify another name.

files number—(Optional) Maximum number of trace files per SNMP subagent. When a trace file (for example, snmpd) reaches its maximum size, it is archived by being renamed to snmpd.0. The previous snmpd.1 is renamed to snmpd.2, and so on. The oldest archived file is deleted.

- Range: 2 through 1000 files

- Default: 10 files

flag flag—Tracing operation to perform. To specify more than one tracing operation, include multiple flag statements:

- all—Log all SNMP events.

- configuration—Log reading of configuration at the [edit snmp] hierarchy level.

- database—Log events involving storage and retrieval in the events database.

- events—Log important events.

- general—Log general events.

- interface-stats—Log physical and logical interface statistics.

- nonvolatile-sets—Log nonvolatile SNMP set request handling.

- pdu—Log SNMP request and response packets.

- policy—Log policy processing.

- protocol-timeouts—Log SNMP response timeouts.

- routing-socket—Log routing socket calls.

- server—Log communication with processes that are generating events.

- subagent—Log subagent restarts.

- timer-events—Log internally generated events.

- varbind-error—Log variable binding errors.

match regular-expression—(Optional) Refine the output to include lines that contain the regular expression.

size size—(Optional) Maximum size, in kilobytes (KB), of each trace file before it is closed and archived.

- Range: 10 KB through 1 GB

- Default: 1000 KB

world-readable | no-world-readable—(Optional) By default, log files can be accessed only by the user who configures the tracing operation. The world-readable option enables any user to read the file. To explicitly set the default behavior, use the no-world-readable option.

snmp-options

Syntax

content_copy zoom_out_map

snmp-options {

backward-traps-only-from-established;

emit-inet-address-length-in-oid;

}

Hierarchy Level

content_copy zoom_out_map

[edit protocols bgp]

Description

Configure the SNMP options and customize the behavior of BGP-related MIBs. By default, SNMP generates traps when a session moves from a higher state to a lower state. For example, from Active to Idle or from Established to Idle. This can result in many uninteresting traps, especially when there are a large number of unconfigured BGP sessions that toggle continuously betweenActive and Idle state. Set the backward-traps-only-from-established option to eliminate the uninteresting traps, and receive backward trap notifications only when transitioning away from the Established state. This can substantially reduce the number of traps sent.

Options

| backward-traps-only-from-established | Limit the generation of traps for backward transitions to session states that are moving from Established state only. |
| --- | --- |
| emit-inet-address-length-in-oid | Emit Length in OID for InetAddress MIB type. This option ensures that SNMP objects related to BGP include the length of internet addresses in their Object Identifier (OID), helping network monitoring tools interpret data correctly and comply with standards for accurate reporting. |

Required Privilege Level

routing—To view this statement in the configuration.

routing-control—To add this statement to the configuration.